WASHINGTON STATE

STEAM LOCOMOTIVE BUILDERS

North Pacific Iron Works   Seattle*

0-4-4   6X10(4)   3 tons   8’ gauge   shipped 8-1-81   $3000

built to the Blackman design with two vertical cylinders powering the pair 		of drivers on each side of a horizontal, return flue boiler   ran on 8” square 		wood rails

Built for Blackman & Howard Mukilteo, WA

to William Gage Mount Vernon, WA 12-82   $2000

gone by 1885, disposition unknown

Seattle Iron Works   Seattle

0-6-0   8X12   10 tons   36” drivers   8’ gauge   shipped 4-3-83

cylinders mounted vertical, one on each side of the 60X96” horizontal 			boiler   ran on 12” wide wood rails

McReavy & Latham-Union River Logging Railroad Co., Belfair, WA

the road was relayed with steel rails to standard gauge in 1885, disposition of 	locomotive unknown

Star Machinery Co.   Seattle

The Pacific Lumber Trade Journal, April 1910, Page 55-”It consists of a pair of 	8X12 upright engines, which are reversible, and a boiler, all mounted on a car.  	There are four drivers, two geared direct to the engines and two driven by chain.  	It is a powerful and inexpensive rig.”  Three known examples:

Sante Fe Logging Co. Arlington, WA

More(sic)-Johnson Shingle Co. Sedro Woolley, WA

Peter Chappel Granite Falls, WA

A locomotive fitting this description is shown in This Was Logging, page 133, Upper.

Vulcan Iron Works   Seattle

0-4-0   self propelled steam donkey

shown in The Timberman, September 1909, page 31

Washington Iron Works   Seattle*@

0-4-4   6X10(4)   8 tons  48” drivers   24” trailers  8’gauge shipped 8-11-83 $3735

Blackman design-see North Pacific Iron Works

Blackman & Howard Mukilteo, WA

“               “       Kamilche, WA 4-85

“               “       Port Susan, WA 3-86

to W.W. Howard Port Susan, WA 5-88

believed here until 1893 default on loan from Port Blakely Mill Co., 			disposition unknown

0-4-4   15 tons   7’ or 7.5’ gauge   shipped 6-3-86   $4765.11

Blackman design-see above, had winch mounted on rear

Blackman Brothers Marysville, WA

to G&O Blackman Tolt (now Carnation), WA 2-89

to Port Blakely Mill Co. for loan default 12-90, disposition unknown

2T Fouts’ Gripwheel   9X12   Standard gauge   3 built 1897

frames and trucks provided by customer

Simpson Logging Co. #1, 4 & 6 Shelton, WA

one burned in forest fire 9-98, all gone by 1905, believed to have been 			converted to 2 drum yarding donkeys

2T Shay type   8X8(2)   Standard gauge   1910

60”X96” return flue horizontal boiler

East & West Lumber Co. Little Valdez (or Valdes) Island, BC

2T Shay Type   8X8(2)   1912

return flue horizontal boiler

C/N 2536   2T Ryan   10X12   31” drivers   7’2” gauge 1915

resembled a 2T version of the Blackman design

locomotive type boiler, oil fired, 10” poles for rails

John Ryan Port Ludlow, WA

parts ordered 1921, disposition unknown.

Several other locomotives were designed but no proof of construction.

This list includes only commercial builders and may well be incomplete.

*see The Blackman Locomotives-Timber Times, Summer 1994, pages 22-28

@see Locomotives by Washington Iron Works-Seattle-Tall Timber Short Lines, July-Aug-Sept 1994, pages 22-27

John A. Taubeneck

2830 NW 56th ST #306

Seattle, WA 98107-4205

January 27, 1996